Assignment – SMPS & UPS POWER SUPPLY

Task - 1. List 3 devices that use SMPS and 2 devices that use UPS

| No. | Device | Type | Reason |
| --- | --- | --- | --- |
| 1 | Desktop Computer | SMPS | It converts 230V AC into different DC voltages (+3.3V, +5V, +12V) required by the computer components. |
| 2 | Laptop Charger | SMPS | It converts AC power into low-voltage DC to safely charge the laptop battery. |
| 3 | LED Television | SMPS | It provides stable DC power efficiently while reducing heat and saving electricity. |
| 4 | Desktop Computer (connected to UPS) | UPS | It provides backup power during a power failure so the computer can be shut down safely. |
| 5 | Wi-Fi Router (connected to UPS) | UPS | It keeps the internet running during short power cuts, preventing connection loss. |

Task - 2. SMPS Internal Block Diagram

SMPS (Switched Mode Power Supply)

AC Input (230V)

│

▼

┌────────────────┐

│ EMI Filter     │

│ (Noise Filter) │

└────────────────┘

│

▼

┌────────────────┐

│ Bridge         │

│ Rectifier      │

└────────────────┘

│

▼

┌────────────────┐

│ Filter         │

│ Capacitor      │

└────────────────┘

│

▼

┌────────────────┐

│ Switching      │

│ Transistor     │

└────────────────┘

│

▼

┌────────────────┐

│ High Frequency │

│ Transformer    │

└────────────────┘

│

▼

┌────────────────┐

│ Output         │

│ Rectifier      │

└────────────────┘

│

▼

┌────────────────┐

│ Output Filter  │

└────────────────┘

│

▼

DC Output

(+3.3V, +5V, +12V)

Task 3

Question:

Take photos or find clear images online of the following power connectors:

- ATX 24-pin Motherboard Connector

- SATA Power Connector

- Molex Connector

- PCIe 6/8-pin Connector

Task 4

Question:

Imagine you are setting up a new gaming PC. List the steps you would follow to connect the SMPS to the motherboard, graphics card, and storage devices, mentioning which specific cables and connectors are used for each.

Steps to Connect the SMPS in a Gaming PC

Step 1: Install the SMPS

Place the SMPS (Power Supply Unit) inside the computer cabinet and secure it with the four screws provided.

Connector Used: None (Physical installation only)

Step 2: Connect the Motherboard

Take the 24-pin ATX power connector from the SMPS and plug it into the 24-pin power socket on the motherboard until it clicks into place.

Cable Used: 24-pin ATX Power Cable

Step 3: Connect the CPU Power

Locate the 8-pin CPU power connector (4+4 pin) and connect it to the CPU power socket near the processor on the motherboard.

Cable Used: 8-pin (4+4) CPU Power Cable

Step 4: Install the Graphics Card

Insert the graphics card (GPU) into the PCIe x16 slot on the motherboard and secure it with screws.

Connector Used: PCIe Slot

Step 5: Connect Power to the Graphics Card

Connect the PCIe 6-pin or 8-pin power connector from the SMPS to the graphics card.

Cable Used: PCIe 6-pin / 8-pin Power Cable

Step 6: Connect the Storage Devices

Install the SSD or HDD in the cabinet and connect the SATA power connector from the SMPS to the storage device.

Cable Used: SATA Power Cable

Step 7: Connect the SATA Data Cable

Connect one end of the SATA data cable to the SSD/HDD and the other end to the SATA port on the motherboard.

Cable Used: SATA Data Cable (Motherboard Cable)

Note: The SATA data cable is not part of the SMPS. It comes with the motherboard.

Step 8: Connect Additional Devices (If Required)

If your PC has RGB controllers, older fans, or IDE devices, connect the Molex power connector.

Cable Used: Molex Connector

Step 9: Check All Connections

Ensure every connector is firmly connected and that the cables are neatly arranged to improve airflow inside the cabinet.

Step 10: Power On the Computer

Connect the AC power cable to the SMPS, switch on the SMPS using the rear power switch, and press the cabinet power button to start the computer.